MQTT Assignment:

Publish

#include <Arduino.h>

#include <PubSubClient.h>

#include <WiFi.h>

#define button_pin 33

#define led_pin 32

const char* ssid = "seamk-titelab";

const char* password = "Sula289tite";

WiFiClient wifiClient;

PubSubClient mqttClient(wifiClient);

const char* mqtt_server = "test.mosquitto.org";

void initWiFi() {

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi ..");

while (WiFi.status() != WL_CONNECTED) {

Serial.print('.');

delay(1000);

}

Serial.println(WiFi.localIP());

}

void reconnect() {

while (!mqttClient.connected()) {

Serial.print("Attempting MQTT connection...");

if (mqttClient.connect("arduinoClient")) {

Serial.println("connected");

} else {

Serial.print("failed, rc=");

Serial.print(mqttClient.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

void setup()

{

Serial.begin(9600);

pinMode(button_pin, INPUT_PULLUP);

initWiFi();

mqttClient.setServer(mqtt_server, 1883);

delay(1500);

}

void loop()

{

if (!mqttClient.connected()) {

reconnect();

}

Serial.println(digitalRead(button_pin));

if (!digitalRead(button_pin))

{

mqttClient.publish("Led_Test", "On");

Serial.println("On");

}

else{

mqttClient.publish("Led_Test", "Off");

Serial.println("Off");

}

//mqttClient.publish("Led_Test", "On");

//Serial.println("julkaistu");

delay(1000);

}

Subscribe

#include <Arduino.h>

#include <PubSubClient.h>

#include <WiFi.h>

// WiFi credentials and MQTT server details

const char* ssid = "seamk-titelab";

const char* password = "Sula289tite";

const char* mqtt_server = "test.mosquitto.org";

// GPIO pin where the LED is connected

#define led_pin 32

WiFiClient wifiClient;

PubSubClient mqttClient(wifiClient);

void initWiFi() {

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

Serial.print("Connecting to WiFi ..");

while (WiFi.status() != WL_CONNECTED) {

Serial.print('.');

delay(1000);

}

Serial.println(WiFi.localIP());

}

void reconnect() {

while (!mqttClient.connected()) {

Serial.print("Attempting MQTT connection...");

// Attempt to connect

if (mqttClient.connect("espClient")) {

Serial.println("connected");

// Subscribe to the topic

mqttClient.subscribe("Led_Test");

} else {

Serial.print("failed, rc=");

Serial.print(mqttClient.state());

Serial.println(" try again in 5 seconds");

delay(5000);

}

}

}

// Callback function that will be executed when a message is received on the subscribed topic

void callback(char* topic, byte* payload, unsigned int length) {

Serial.print("Message arrived in topic: ");

Serial.println(topic);

Serial.print("Message:");

String message;

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

Serial.println(message);

Serial.println("-----------------------");

// Turn the LED ON or OFF based on the message content

if (message == "On") {

digitalWrite(led_pin, HIGH); // Turn the LED on

} else if (message == "Off") {

digitalWrite(led_pin, LOW); // Turn the LED off

}

}

void setup() {

Serial.begin(9600);

pinMode(led_pin, OUTPUT);

initWiFi();

mqttClient.setServer(mqtt_server, 1883);

mqttClient.setCallback(callback);

}

void loop() {

if (!mqttClient.connected()) {

reconnect();

}

mqttClient.loop();

}

Pictures: